Quick guide to Pair the FM-201 device
*Some user reported they can not get the Bluetooth connected with their phones. The reason is that they did not pair the Bluetooth with phone initially.
All Bluetooth devices need to pair first before they can connect with phones to use.
· Turn your mobile’s Bluetooth search or add new device mode on, some mobiles include iPhone Bluetooth needs to be in discoverable mode 
· Press and hold the power button to turn on the device till you see “Power on” message displayed on the top left side of the device after the welcome messages, and please keep holding, you will see “Power on” message changing to “Pairing” message on the top left side , 

· You will find that the Bluetooth logo [image: image1.emf] is flicking quickly. Your mobile starts to search the available Bluetooth device. Your mobile should find the “Car kit” tagged Bluetooth device displayed on your mobile Bluetooth section. 
· Your mobile will pop up message to ask the permission to pair with FM201. To pair your mobile phone with this device, enter the access code “0000” to pair if necessary. 
· After paired successful, your mobile can connect or disconnect with FM201 as you like
· If you delete the FM201 car kit from your phone, remember that you need to follow above process to pair it again in order to connect it again. 
